CSI Format Specification for APEX™ railings – with Infill

Part 1 - General:

  - Section Includes

Pre-engineered Apex™ powder coated aluminum and stainless steel Railing System. All drawings general and supplementary conditions including division one specifications apply to this section.

  - Related Sections

Division 5: Ornamental Handrails and Railings.

1.3	Product References and Design Requirements

1.3.1    Principle items specified in this section are:

a.  Stainless steel and Stainless/wood combination handrails.

b.  Powder coated Aluminum guardrails or other ornamental barrier railings.

c.  Powder coated Aluminum mounting plates for surface or side mounting.

1.3.2	Design requirements are based on IBC/IRC and ADA standards:

a.  Guardrails and handrails shall meet or exceed all applicable building codes.

b.  Railings shall have high strength stainless steel to comply with structural requirements with an appropriate safety margin.

c.  All internal members shall be stainless steel or aluminum to eliminate the possibility of rust.

d.  Prevent galvanic action and other forms of corrosion by insulating metals and other materials from direct contact with incompatible materials.

1.3.3	Work Included

Provide all materials, labor and equipment necessary to fabricate and completely install 		handrails, guardrails, and other railing options as shows on drawings or specified herin.

1.3.4	Definitions

Terms and definitions from ASTM E985 and ISO/TC 59 for railing related items apply to 		this section.

1.4	System Performance Requirements

1.4.1	Railings shall meet or exceed the requirements of all applicable building codes.

1.4.2	Railings shall have high strength aluminum in order to comply with 1.41 with 			adequate safety margin.

1.4.3	All internal members shall be aluminum, stainless steel, to eliminate the possibility of 		rust.

1.4.4	Prevent galvanic action and other forms of corrosion by insulating metals and other 		materials from direct contact with incompatible materials.

  - Submittals

    - Shop Drawings for architectural approval, showing fabrication and installation of 	handrails and railings including plans, elevations, sections, details of components and 	attachments to other units of work.

    - Product data for aluminum, stainless steel/wood products to be supplied by the mfg.

    - Structural computations or test data/evaluations, material properties, PE (professional 	engineering) calculations signed/sealed in the State of the project, and other information 	needed to ensure satisfactory structural compliance to applicable building codes to be 	supplied by the manufacturer, based on final fabrication drawings and documents.

    - Maintenance instructions: Provide manufacturer‘s maintenance and cleaning 	instructions.

    - Warranty: Provide manufacturer‘s warranty effective from completion of work.

    - Initial selection

Provide samples complete to demonstrate powder coated aluminum color and finish, stainless steel grade and finish and or wood type and finish. Nylon components to be includes if specified, color as indicated.

    - Final verification

Qualification data for authorized installers specified in Quality Assurance is to 	demonstrate their capabilities and experience. Include list of completed projects with 	project and architect names.

  - Quality Assurance

    - Single Source Responsibility: Materials shall be supplied by HDI Railing Systems, 	3905 Continental Drive, Columbia PA 17512 (Tel: 717-285-4088 Fax 717-285-5083).

www.hdirailings.com Regionally based Sales Managers available.

    - Execution tolerance plus/minus 5/64” (2 mm).

  - Storage

1.7.1    Store handrails and railing systems in clean, dry location, away from uncured concrete 		and masonry, protected against damage of any kind.

1.7.2	Materials must be kept in original packing until installation.

1.7.3	Materials to be stored at not lower than -40°C (-104°F) or higher than 100°C (212°F).

1.8       Project Conditions

1.8.1	All measurements for handrails and railings should be taken from construction site 		elements to which railings are to fasten. This information to be recorded on final shop 		drawings.

1.8.2	Coordinate fabrication schedule with construction progress to avoid delay of work

Part 2 - Product:

  - Manufacturer

2.1.1.	Mechanically fastened Stainless steel railing systems

Manufacturer shall be HDI Railing Systems, an established U.S. manufacturer of a 		custom pre-engineered, mechanically fastened guardrail and handrail systems, in strict 		compliance with all technical requirements of the drawings and specifications. 			Miscellaneous metal fabricators/suppliers will not be acceptable. This standard is based 		on HDI Railing Systems, 3905 Continental Drive, Columbia PA 17512 (Tel. 			717.285.4088 Fax. 717.285.5083).

2.1.2	Substitutions:  Not permitted.

2.2       Materials for Guardrails and Handrail System

    - All rails and other tubular components shall be constructed using the following:

          - Aluninum grade 6063-T6, Stainless steel grade AISI, type 304/304L or 316/316L; surface to be 240 grain/grit finish; tubes 1-1/2” (38mm) outside diameter by 5/64” (2 mm) wall thickness.

        - Optional natural wood handrails and top rails to be connected to stainless steel or wood with saddle connectors and stainless steel connectors to posts: Mitered joints required by changes in direction require stainless steel true bar inserts  wood. Standard wood  types are available in natural beech, birch or maple, other wood types available upon request. All custom stains are subject to customer approval and require customer samples. Wood to be continuous throughout. Returns to wall or floor that require stainless steel bends should have transitions that are smooth and without burrs

    - All posts and other components shall be constructed using the following:

                - Posts and pickets to be type 6063-T6 aluminum 2“ (50mm) by 1/2“ (13mm).Hardware attachment surface to be through post to base. All steel anchors and bolts to be powder coated to required color (standard listed herein) or custom RAL® color:

49-66230	Light Bronze

49-50024	Forest Green

39-800	20	Cobalt Black

39-60060	Antique Bronze

49-11280	Off White

49-40033	Tanzanite Blue

349-79680	Slate Grey

49-37900	Ruby Red

33-9740	Stardust Silver

      - Surface and side mount fastening plates arnd clamps to be made from alumminum grade 6063-T6 and powder coated to match railings.

    - Fastening bolts to be stainless steel or other high strength material as determined by 	engineering requirements.

      - Infill Pickets to be through bolted to upper an lower horizontal rails per manufacturers (HDI Railing Systems)

2.3	Stainless steel picket rails

2.3.1	Stainless steel solid vertical picket rails 3/8“ (9.5 mm) supported by upper and 			lower rails 5/8“ (15mm * 2mm). Picket rails on approx. 4“ (100 mm) centers, gaps 		between pickets and adjacent posts to be equalized depending on required rail length 		and site conditions not to exceed 4“ (100 mm). Brushed finish #6 on vertical picket, 		upper and lower supports to be polished radially. Clamping knobs and fixtures to be 		stainless steel finished to match.

2.4	Fasteners

2.4.1	Anchors shall be fabricated from stainless steel or other materials as determined by engineering requirements with capability to sustain, without failure, load imposed within a safety factor of 4, as determined by testing per ASTM E488.

2.5      Fabrication

    - Fabricate railing system for compliance with structural requirements of applicable 		code.

    - Clearly mark units for assembly and for coordination with shop drawings.

2.5.3	Aluminum posts, pickets, bases and Stainless steel tubing cuts shall be square, without burrs and where exposed, to produce smooth rigid and hairline joints.

Part 3 – Execution:

3.1       Examination

3.1.1	Do not begin installation until substrates have been properly prepared.

3.1.2	Verify that reinforcement and anchoring devices are the correct type, have been located correctly, and have been installed properly.

3.1.3	If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

3.2	Preparation

Provide information on fastening point locations for posts where necessary to relevant parties.

3.3	Installation

Installation shall be by HDI Railing Systems or a qualified, authorized representative of the manufacturer. HDI Railing Systems, 3905 Continental Drive, Columbia PA 17512 (Tel: 717-285-4088 Fax 717-285-5083).

    - Installation must be in accordance with standard or non-standard, yet applicable details

(instructions) included on installation/shop drawings provided by HDI Railing Systems.

3.3.2	Install components plumb and in-line, accurately fitted, free from distortion or defects and securely anchored to structure.

3.3.3	Provide anchors, plates, angles, etc., necessary for connecting railings to structure.

3.3.4 	Any and all field welding shall be by a certified welder.

3.3.5	Access for anchors that require through bolting either vertically or horizontally to be made available through General Contractor.

Erection tolerances

    - Maximum variation from plumb shall be 1/4" (6 mm).

    - Maximum offset from true alignment for every 50-foot of railing shall be 1/4" (6 mm), 	non-accumulative.

3.4	Cleaning and Protection

    - Remove manufacturer's protective coverings from exposed surfaces after installation.

    - Railings shall be cleaned, including infill, by contractor to the satisfaction of the owner.

    - Wipe with moistened cloth only.  Do not use cleaning agents with abrasive or 	acid/alkaline content.

    - General contractor to provide protective covering on handrails and guardrails if 	construction is not yet finished in the area where the railings are installed.

  - Correction of deficiencies

All deficiencies in work and/or items not meeting specified requirements shall be corrected in

order to meet specification requirements at no additional cost to owner.

End of Section