CSI Format Specification for KOTO™ Square cable railings – with LED options

Part 1 - General:

  - Section Includes

Pre-engineered KOTO™ Square post stainless steel Railing System. All drawings, general and

supplementary conditions including division one specifications apply to this section.

  - Related Sections

Division 5: Ornamental Handrails and Railings

1.3	Product References and Design Requirements

1.3.1    Principle items specified in this section are:

a.         Stainless steel and Stainless/wood combination handrails.

b.         Stainless steel and steel guardrails or other ornamental barrier railings.

c.	Stainless steel cables and cable hardware.

d.	Stainless steel rail with LED light insert.

1.3.2	Design requirements are based on IBC/IRC and ADA standards:

a  Guardrails and handrails shall meet or exceed all applicable building codes.

b.  Railings shall have high strength stainless steel to comply with structural requirements with an appropriate safety margin.

c.  Railings including top rail, end posts, mid posts, cable and cable hardware shall be designed to comply with structural requirements of the building code requirements with an appropriate safety margin.

d.  Railing and cable hardware shall be designed to withstand loads encountered without excessive deflection or distortion when cables are tensioned to required amounts to conform to building codes.

e.  All internal members shall be stainless steel, aluminum, or nylon to eliminate the possibility of rust.

f.  Prevent galvanic action and other forms of corrosion by insulating metals and other materials from direct contact with incompatible materials.

1.3.3	Work Included

Provide all materials, labor and equipment necessary to fabricate and completely install 		handrails, guardrails, infill panels, and other railing options as shown on drawings or 			specific herein.

1.3.4	Definitions

Terms and definitions from ASTM E985 and ISO/TC 59 for railing related items apply to 		this section.

1.4	System Performance Requirements

1.4.1	Railings shall meet or exceed the requirements of all applicable building codes.

1.4.2	Railings shall have high strength stainless steel in order to comply with 1.4.1 with 			adequate safety margin.

1.4.3	All internal members shall be stainless steel, aluminum, nylon, or wood to eliminate the possibility of rust.

1.4.4	Prevent galvanic action and other forms of corrosion by insulating metals and other 			materials from direct contact with incompatible materials.

  - Submittals

    - Shop Drawings for architectural approval, showing fabrication and installation of 	handrails and railings including plans, elevations, sections, details of components and

attachments to other units of work.

    - Product data for stainless steel/wood products to be supplied by the manufacturer.

    - Structural computations or test data/evaluations, material properties, PE (professional

engineering) calculations signed/sealed in the State of the project, and other information needed to ensure satisfactory structural compliance to applicable building codes to be supplied by the manufacturer, based on final fabrication drawings and documents.

    - Maintenance instructions: Provide manufacturer‘s maintenance and cleaning

instructions.

    - Warranty: Provide manufacturer‘s warranty effective from completion of work.

    - Initial selection

Provide sample of railing including post material, top rail, and cables demonstrating

attachment hardware and method of attachment.

    - Final verification

Qualification data for authorized installers specified in Quality Assurance is to

demonstrate their capabilities and experience. Include list of completed projects with 	project and architect names.

  - Quality Assurance

    - Single Source Responsibility: Materials shall be supplied by HDI Railing Systems, 	3905 Continental Drive, Columbia PA 17512 (Tel: 717-285-4088 Fax 717-285-5083).

www.hdirailings.com Regionally based Sales Managers available.

    - Execution tolerance plus/minus 5/64” (2 mm).

  - Storage

1.7.1    Store handrails and railing systems in a clean, dry location, away from uncured concrete 			and masonry, protected against damage of any kind.

1.7.2	Materials must be kept in their original packaging until installation.

1.8       Project Conditions

1.8.1	All measurements for handrails and railings should be taken from construction site 			elements to which railings are to fasten. This information to be recorded on final shop 			drawings.

1.8.2	Coordinate fabrication schedule with construction progress to avoid delay of work

Part 2 - Product:

  - Manufacturer

2.1.1.	KOTO™ Stainless steel cable railing systems

Manufacturer shall be HDI Railing Systems, an established U.S. manufacturer of a 			custom pre-engineered, mechanically fastened guardrail and handrail systems, in strict 		compliance with all technical requirements of the drawings and specifications. 				Miscellaneous metal fabricators/suppliers will not be acceptable. This standard is based 		on HDI Railing Systems, 3905 Continental Drive, Columbia PA 17512 (Tel. 				717.285.4088 Fax. 717.285.5083).

2.1.2	Substitutions:  Not permitted.

2.2       Materials for Guardrails and Handrail System

    - All handrails and other tubular components shall be constructed using the following:

2.2.1.1   Stainless steel grade type AISI 304/304L or 316/316L; surface to be 240 			grain/grit finish; tubes 1-1/2” (38mm) outside diameter by 5/64” (2 mm) wall 			thickness.

      - Optional natural wood handrails and top rails to be connected to stainless steel

saddles. Mitered joints required by changes in direction require stainless steel true bar to be inserted into the underside of the wood. Standard wood types are available in natural beech, birch or maple, other wood types available upon request. All custom stains are subject to customer approval and require customer samples. Wood to be continuous throughout. Returns to wall or floor that require stainless steel bends should have transitions that are smooth and without burrs.

      - Powder coating options available upon request.

    - All posts and other components shall be constructed using the following:

2.2.2.1   Stainless steel grade AISI type 304 or 316, surface to be 240 grain/grit finish; 			tubes 2”x 2“(51mm) diameter by 0.125“ (3.2 mm)wall thickness.

2.2.2.2    Stainless steel grade AISI type 304 or 316, surface to be 240 grain/grit finish 				for: end caps at top of posts. Hardware for handrail attachment to match finish of 			posts.

      - Stainless steel grade AISI type 304 or 316, surface to be 240 grain/grit (#6) 	finish for post fastening base plate.

      - Powder coating option available upon request

    - Fastening bolts to be stainless steel or other high strength material as determined by 	engineering requirements.

    - Exterior and aggressive environments require stainless steel grade AISI 316 or 316L to 	minimize maintenance requirement; surface to be 240 grain/grit (#6) finish.  Powder

coating options available upon request.

  - Stainless steel cable rails, cable hardware and woven cable infill mesh

2.3.1	Cables

      - Material:  7x19 Type 316 stainless steel stranded

      - Diameter: 3/16 inch (5 mm) diameter cable with a minimum breaking strength of

13KN (2,923 pounds)

      - Finish: mill

- ** NOTE TO SPECIFIER ** Select orientation.  Delete three of the following four paragraphs.

      - Orientation:  As indicated on the Contract Drawings

- ** NOTE TO SPECIFIER **  Select spacing requirement.  Delete one of the next two paragraphs.

2.3.3.5     Spacing:  As indicated on the Contract Drawings (max. 3-1/8” - 79mm)

2.3.2   Cable Hardware Components

2.3.2.1    Material:  Stainless steel, AISI 316L; Swaged hardware at end of cable

2.3.2.2    Hardware concealed inside end posts where practical.

2.3.2.3    To comply with HDI Railing Systems’ design criteria.

2.3.2.4    Types of Fittings: Radius head internal thread adjuster; External thread

swaged end

2.4     LED lighting inserts (LED components subject to change; contact Manufacturer for most current LED specifications)

    - Stainless steel rail with LED light strip inserts grade AISI type 304 or type 316; surface to be # 6 finish (240 grain/grit); tubes 1.5” (38mm) outside diameter by .075“ to .090” (1.5 mm – 2.3mm) wall thickness.

    - LED light strip inserts with a beam spread of 45 degrees to 90 degrees, or asymmetrical beam spread of 28 degrees and 65 degrees off nadir. Other beam spreads available; please contact the manufacturer.

    - Standard clear lens; frosted lens optional.

    - Light output at 42“guard height, lumens per ft as indicated, ranging from 36.8 to 616 lumens per ft dependent on standard or high output and symmetrical or asymmetrical.

    - Color temperature as indicated, ranging from 2700K (Kelvin) to 6500K, dependent on lumens per ft and beam spread required.

    - Electrical supply for LED strip inserts to be 120-277Volt supply for 24-volt remote power supply. (Power supply range 20-240watts)

    - Transition sleeves for intermediate supports, backing strips for LED light strips, slotted tube inserts and mounting clips by HDI provided as required.

    - Power supply low voltage dimmers are available.

  - HDI Puck LED (LED components subject to change; contact Manufacturer for most current LED specifications)

    - Stainless steel rail with LED puck fixtures grade AISI type 304 or type 316; surface to be #6 brushed finish (240 grain/grit); tubes 1.5“ (38mm) outside diameter by .075“ to .090“ (1.5mm – 2.3mm) wall thickness.

    - Individual LED puck fixtures come in 316 stainless steel grades. 18mm diameter x 24mm depth.

    - LED puck fixtures power consumption is 1.2W per fixture and are powered by DC24V power.  Each fixture is IP67 waterproof rated and has 3M waterproof internal connectors.  Dimmable fixtures available upon request.

    - Standard clear lens.

    - Standard beam angle is 60 degrees symmetric, customized beam angles available upon request.  Asymmetric options are available.   Please contact manufacturer.

    - Color temperature as indicated, ranging from 2700K (Kelvin) to 6000K.  Other solid color options are available upon request such as Blue, Green, and Red.

    - Electrical supply for LED puck fixtures to be 120-277Volt supply for 24-volt remote power supply. (Power supply range 20-240 watts)

    - Power supply low voltage dimmers are available.

    - Low voltage wires for use of connecting fixtures inside of the handrail provided by manufacturer.  Installation of wires and puck fixtures are required in the field during installation process (not pre-assembled in HDI factory).  Pre-drilled and tapped holes for fixture mounting into the handrail can also be provided by manufacturer in advance.  For curved or spiral applications, holes are required to be drilled and tapped in the field.  See manufacturer for more information.

2.6	Fasteners

Anchors shall be fabricated from stainless steel or other materials as determined by engineering requirements with capability to sustain, without failure, load imposed within a safety factor of 4, as determined by testing per ASTM E488.

2.7    	  Fabrication

2.7.1   Fabricate railing system for compliance with structural requirements of applicable code.

2.7.2   Pre-assemble railings prior to shipping to greatest extent possible to ensure QC.

Disassemble units and pack as appropriate for shipping and handling requirements.  Clearly mark crates and boxes for re-assembly by location and for coordination with shop drawings.

2.7.3  Stainless steel tubing cuts shall be square, without burrs and where exposed, rounded 			to produce smooth rigid and hairline joints.

Part 3 – Execution:

3.1       Examination

3.1.1	Do not begin installation until substrates have been properly prepared.

3.1.2	Verify that reinforcement and anchoring devices are the correct type, have been located correctly, and have been installed properly.

3.1.3	If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

3.2	Preparation

Provide information on fastening point locations for posts where necessary to relevant parties.

3.3	Installation

Installation shall be by HDI Railing Systems or a qualified, authorized representative of the manufacturer. HDI Railing Systems, 3905 Continental Drive, Columbia PA 17512 (Tel: 717-285-4088 Fax 717-285-5083).

    - Installation must be in accordance with standard or non-standard, yet applicable details

(instructions) included on installation/shop drawings provided by HDI Railing Systems.

3.3.2	Install components plumb and in-line, accurately fitted, free from distortion or defects and securely anchored to structure.

3.3.3	Provide anchors, plates, angles, etc., necessary for connecting railings to structure.

3.3.4 	Any and all field welding shall be by a certified welder.

3.3.5	Access for anchors that require through bolting either vertically or horizontally to be made available through General Contractor.

Erection tolerances

    - Maximum variation from plumb shall be 1/4" (6mm).

    - Maximum offset from true alignment for every 50-foot of railing shall be 1/4" (6 mm), 	non-accumulative.

3.4	Cleaning and Protection

    - Remove manufacturer's protective coverings from exposed surfaces after installation.

    - Railings shall be cleaned, including infill panels, by contractor to the satisfaction of the owner.

    - Wipe with moistened cloth only.  Do not use cleaning agents with abrasive or acid/alkaline content.

    - General contractor to provide protective covering on handrails and guardrails if construction is not yet finished in the area where the railings are installed.

  - Correction of deficiencies

All deficiencies in work and/or items not meeting specified requirements shall be corrected in

order to meet specification requirements at no additional cost to owner.

End of Section