CSI Format Specification for Optik Shoe railings – with LED options

Part 1 - General:

  - Section Includes

Pre-engineered Optik glass and stainless steel Railing System. All drawings, general and supplementary conditions including division one specifications apply to this section.

  - Related Sections

Division 5: Ornamental Handrails and Railings.

1.3	References and Design Requirements

1.3.1    Principle items specified is this section are:

a.         Stainless steel or Stainless/wood combination handrails.

b.	Stainless steel mounting hardware.

- Tempered-laminated glass structural infill panels.

- Stainless steel rail with LED light insert.

1.3.2    Design requirements are based on IBC/IRC and ADA standards:

1.3.2.1	   Guardrails and handrails shall meet or exceed all applicable building codes.

1.3.2.2	   Railings shall have high strength stainless steel to comply with structural

requirements with an appropriate safety margin.

1.3.2.3	9/16“ thick structural glass panels shall be fully tempered with a SGP laminated interlayer and a required attached handrail or cap rail, unless local code otherwise allows.

1.3.2.4	  13/16“ structural glass panels shall be fully tempered with a SGP laminated interlayer and an attached handrail or cap rail.

1.3.2.5	  All internal members shall be stainless steel or aluminum to eliminate the possibilty of rust.

1.3.2.6	  Prevent galvanic action and other forms of corrosion by insulating metals and other

materials from direct contact with incompatible materials.

1.3.3	   Work Included

Provide all materials, labor and equipment necessary to fabricate and completely 		install handrails, guardrails, infill panels, and other railing options as shows on 			drawings or specific herin.

1.3.4    Definitions

Terms and definitions from ASTM E985 and ISO/TC 59 for railing related items apply 		to this  section.

1.4    System Performance Requirements

1.4.1	Railings shall meet or exceed the requirements of all applicable building codes.

1.4.2	Railings shall have high strength stainless steel in order to comply with 1.4.1 with 		adequate safety margin.

1.4.3	All internal members shall be stainless steel, nylon or wood to eliminate the possibility 		of rust.

1.4.4	Prevent galvanic action and other forms of corrosion by insulating metals and other 		materials from direct contact with incompatible materials.

  - Submittals

    - Shop Drawings for architectural approval, showing fabrication and installation of handrails and railings including plans, elevations, sections, details of components and attachments to other units of work.

    - Product data for stainless steel/wood products to be supplied by the manufacturer.

    - Structural computations or test data/evaluations, material properties, PE (professional engineering) calculations signed/sealed in the State of the project, and other information needed to ensure satisfactory structural compliance to applicable building codes to be supplied by the manufacture, based on final fabrication drawings and documents.

    - Maintenance instructions: Provide manufacturer‘s maintenance and cleaning instructions.

    - Warranty: Provide manufacturer‘s warranty effective from completion of work.

    - Initial selection

Provide 6“ long handrail samples complete with supports and rosette covers to demonstrate stainless steel grade and finish. Nylon components to be included if specified, color as indicated.

    - Final verification

Qualification data for authorized installers specified in Quality Assurance is to demonstrate their capabilities and experience. Include list of completed projects with project and architect names.

  - Quality Assurance

    - Single Source Responsibility: Materials shall be supplied by HDI Railing Systems, 3905 Continental Drive, Columbia PA 17512 (Tel: 717-285-4088 Fax 717-285-5083).

    - Execution tolerance plus/minus 5/64” (2 mm).

  - Storage

    - Store handrails and railing systems in a clean, dry location, away from uncured concrete and masonry, protected against damage of any kind.

1.6.2    Materials must be kept in their original packing until installation.

1.8     Project Conditions

1.8.1	All measurements for handrails and railings should be taken from construction site

elements to which railings are to fasten. This information to be recorded on final shop

drawings.

1.8.2	Coordinate fabrication schedule with construction progress to avoid delay of work

Part 2 - Product:

  - Manufacturer

2.1.1	Mechanically fastened Structural Glass and Stainless steel railing systems

Manufacturer shall be HDI Railing Systems, a U.S. manufacturer of a custom pre-engineered, mechanically fastened guardrail and handrail system, in strict compliance with all technical requirements of the drawings and specifications. Miscellaneous metal fabricators/suppliers will not be acceptable. This standard is based on HDI Railing Systems, 3905 Continental Drive, Columbia PA 17512 (Tel. 717.285.4088 Fax. 717.285.5083).

2.2       Materials:   Guardrails and Handrail System

2.2.1	All rails and other tubular components shall be constructed using the following:

a.	Handrails to be Stainless steel grade UNS 1.4305, type 304; surface to be 240 	grain/grit finish; tubes 1-1/2” (38mm) outside diameter by 5/64” (2 mm) wall thickness.

b. 	Handrail supports to be Stainless steel grade UNS 1.4305, type 304;finish to match 	handrail.

c. 	Metal Cap to be stainless steel satin finish (architect to select type and size)

Square section, 1“ by 1-5/16“, 1-1/2“ or 2“ square

Round section,  diameters 1-1/2“, 1.66“, 2“, 2-1/2“ or 3“

b. 	Optik Disc lock™ system includes plastic inserts to isolate glass from metals and a 	stainless Disc lock™ mechanism to secure glass in the aluminum shoe base.

c.	Shoe Base:

i.	Profile: Rectangular cross-section.2-3/4 inches (63.5 mm) wide by 4-1/8 inches 		(104.7 	mm) high

ii.	Material: Aluminum 6063 – T52

iii. 	Finish: Mill finish

iv.	Base Cladding: Brushed finish sheet metal cladding added to exposed shoe 		base sections. Adhere with double-sided tape and/or silicone adhesive. 			Provide end caps where ends of shoe base sections are exposed.

d.	Optional natural wood handrails and top rails to be connected to posts using stainless steel saddles on top of glass attached handrail brackets: Mitered joints required by changes in direction require stainless steel true bar to be inserted into the underside of the wood. Standard wood  types are available in natural beech, birch or maple, other wood types available upon request. All custom stains are subject to customer approval and require customer samples. Wood to be continuous throughout. Returns to wall or 	floor that require stainless steel bends should have transitions that are 			smooth and without burrs.

2.2.3	Fastening for shoe mounting method structural glass to be as specified in Glass products section (2.4) base to be Aluminum alloy 6063 - T52.

    - Fastening bolts to be stainless steel or other high strength material as determined by engineering requirements.

    - Neoprene gasket material to be used at all through bolt connections to ensure seperation between metal and glass.

2.3       Glass products, glazing and infill materials.

    - Tempered-Laminated glass:  Provide fully tempered safety glass with polished edges

and dubbed (blunt) corners complying with ASTM C1048.  Kind FT (fully tempered), condition A (un-coated).  Types 1 (transparent glass, flat), quality Q3 (glazing select), class, 	thickness and manufacturing process as indicated below. Tempered-laminated glass 	infill panels are required in railings where there is access to areas below the railing 	(check local codes).

2.3.2	Clear glass: Class 1 clear

2.3.3	Tinted glass: Class 2 (tinted heat absorbing and light reducing). Manufacturer’s 			standard tint color indicated below:

a.	Bronze, Gray, or other as indicated

b.	Low iron

c.	Custom pattern, art glass, etch, edges or surface, laminate or embeds, as 		specified herein.

    - Allowable thickness either:

2.3.4.1    9/16“ thick structural glass panels shall be fully tempered with SGP laminated interlayer and a required attached handrail or cap rail to be noted on final shop drawings, unless local code otherwise allows.

2.3.4.2   13/16“ structural glass panels shall be fully tempered with SGP laminated interlayer and an attached handrail or cap rail to be noted on final shop drawings.

2.3.5	Manufacturing process: Manufacture fully tempered glass by horizontal (roller hearth) 		process with roll wave distortion parallel with bottom edge of glass as installed.

    - Optik Shoe structural glass railing to be designed for compliance with appropriate 		building codes. Structural computations showing material properties and other 			information needed to ensure satisfactory structural compliance to applicable building 		codes to be supplied by the manufacturer – based on final fabrication drawings and 		documents, anchoring type, spacing and glass thickness.

    - Laminated Interlayer:  PVB for interior applications, SGP for exterior applications

2.4	LED lighting inserts (LED components subject to change; contact Manufacturer for 	most 	current LED specifications)

2.4.1	Stainless steel rail with LED light strip inserts grade UNS 1.4305, type 304 or type 		316; surface to be # 6 finish (240 grain/grit); tubes 1.5” (38mm) outside 			diameter by .075“ to .090” (1.5 mm – 2.3mm) wall thickness.

2.4.2	LED light strip inserts with a beam spread of 45 degrees to 90 degrees, or 			asymmetrical beam spread of 28 degrees and 65 degrees off nadir. Other beam 		spreads available; contact the manufacturer.

2.4.3	 Standard clear lens; frosted lens optional.

2.4.4	Light output at 42“guard height, lumens per ft as indicated, ranging from 36.8 to 616 		lumens per ft dependent on standard or high output and symmetrical or asymmetrical.

2.4.5	Color temperature as indicated, ranging from 2700K (Kelvin) to 6500K, dependent on 		lumens per ft and beam spread required.

2.4.6	Electrical supply for LED strip inserts to be 120-277Volt supply for 24-volt remote 		power supply. (Power supply range 20-240watts)

2.4.7	Transition sleeves for intermediate supports, backing strips for LED light strips, slotted 		tube inserts and mounting clips by HDI provided as required.

2.4.8	Power supply low voltage dimmers available.

  - HDI Puck LED (LED components subject to change; contact Manufacturer for

most current LED specifications)

    - Stainless steel rail with LED puck fixtures grade AISI type 304 or type 316;

surface to be #6 brushed finish (240 grain/grit); tubes 1.5“ (38mm) outside diameter by .075“ to .090“ (1.5mm – 2.3mm) wall thickness.

    - Individual LED puck fixtures come in 316 stainless steel grade. 18mm diameter x

24mm depth.

    - LED puck fixtures power consumption is 1.2W per fixture and are powered by

DC24V power. Each fixture is IP67 waterproof rated and have 3M waterproof internal connectors. Dimmable fixtures available upon request.

    - Standard clear lens.

    - Standard beam angle is 60 degrees symmetric, customized beam angles

available upon request.  Asymmetric options are available.   Please contact manufacturer.

    - Color temperature as indicated, ranging from 2700K (Kelvin) to 6000K.  Other

solid color options available upon request such as Blue, Green, and Red.

    - Electrical supply for LED puck fixtures to be 120-277Volt supply for 24-volt.

remote power supply. (Power supply range 20-240 watts).

    - Power supply low voltage dimmers are available.

    - Low voltage wires for use of connecting fixtures inside of the handrail provided

by manufacturer.  Installation of wires and puck fixtures are required in the field during installation process (not pre-assembled in HDI factory).  Pre-drilled and tapped holes for fixture mounting into the handrail can also be provided by manufacturer in advance.  For curved or spiral applications, holes are required to be drilled and tapped in the field.  See manufacturer for more information.

2.6	Fasteners

Types and sizes indicated in shop drawings.

- For concrete attachment, hole size in base shoe is to be 9/16” (14.3 mm), counter bore 7/8” (22.2 mm) x depth ½” (12.7 mm), center-to-center spacing of holes is 12” (304.8mm). Use Power Fasteners 3/8” x 4” Wedge-Bolts Part # WBA38X4 with Wedge-Bolt Washer Part # WBAW38.

- For steel attachment, hole size in base shoe is to be 9/16” (14.3 mm), counter bore 7/8” (22.2 mm) x depth ½” (12.7 mm), center-to-center spacing of holes is 12” (304.8mm). Use ½” – 13 x 1 stainless steel socket head cap screw Part # SHCS12X1.

- Sill Angles for Tempered Glass Railing Assemblies: Steel angle profiles conforming to ASTM A 36, with anchoring devices, sizes indicated in shop drawing of section 05522, drilled and tapped for fastener types, sizes, and spacing indicated.

2.5.1	Anchors shall be fabricated from stainless steel or other materials as determined by 		engineering requirements with capability to sustain, without failure, load imposed 		within a safety factor of 4, as determined by testing per ASTM E488.

2.7   	Fabrication

2.6.1	Fabricate railing system for compliance with structural requirements of applicable 		code.

2.6.2	Pre-assemble railings prior to shipping to greatest extent possible to ensure QC.  Disassemble units and pack as appropriate for shipping and handling requirements.  Clearly mark crates and boxes for re-assembly by location and for coordination with shop drawings.

2.6.3   	Stainless steel tubing cuts shall be square, without burrs and where exposed, rounded 		to produce smooth rigid and hairline joints.

Part 3 – Execution:

3.1       Examination

3.1.1	Do not begin installation until substrates have been properly prepared.

3.1.2	Verify that reinforcement and anchoring devices are the correct type, have been 		located correctly, and have been installed properly.

3.1.3	If substrate preparation is the responsibility of another installer, notify Architect of 		unsatisfactory preparation before proceeding.

3.2	Preparation

Provide information on fastening point locations for anchors or posts where necessary to 	relevant parties.

  - Installation

Installation shall be by HDI Railing Systems or a qualified, authorized representative of the manufacturer. HDI Railing Systems, 3905 Continental Drive, Columbia PA 17512 (Tel: 717-285-4088 Fax 717-285-5083).

    - Installation must be in accordance with standard or non-standard, yet applicable details (instructions) included on installation/shop drawings provided by HDI Railing Systems.

3.3.2	Install components plumb and in-line, accurately fitted, free from distortion or defects 		and securely anchored to structure.

3.3.3	Provide anchors, plates, angles, etc., necessary for connecting railings to structure.

3.3.4 	Any and all field welding shall be by a certified welder.

3.3.5	Access for anchors that require through bolting either vertically or horizontally to be made available through General Contractor.

Erection tolerances

    - Maximum variation from plumb shall be 1/4".

    - Maximum offset from true alignment for every 50-foot of railing shall be 1/4", non-	accumulative.

3.4	Cleaning and Protection

    - Remove manufacturer's protective coverings from exposed surfaces after installation.

    - Railings shall be cleaned, including infill panels, by contractor to the satisfaction of the 	owner.

    - Wipe with moistened cloth only.  Do not use cleaning agents with abrasive or 	acid/alkaline content.

    - General contractor to provide protective covering on handrails and guardrails if 	construction is not yet finished in the area where the railings are installed.

  - Correction of deficiencies

All deficiencies in work and/or items not meeting specified requirements shall be corrected in

order to meet specification requirements at no additional cost to owner.

End of Section